Quality specification of RUE “Belmedpreparaty” on API

Sodium Sulfobutyl ether beta-Cyclodextrin

| No. | Items | Requirements of in-house technical documentation of the Republic of Belarus |
| --- | --- | --- |
| 1. | Description | White or almost white crystalline powder |
| 2. | Solubility | Freely soluble in water, sparingly soluble in methanol, practically insoluble in ethanol 96%, in hexane, in butanol, in acetonitrile, 2-propanol and in ethyl acetate |
| 3. | Identification: - Sodium Sulfobutyl ether beta-Cyclodextrin - Sodium | 1. The IR spectrum of the test sample should match the standard of the Sodium Sulfobutyl ether beta-Cyclodextrin 2. On the chromatogram of the test solution obtained by quantification, the retention time of the peak of the sodium salt of sulfobutyl ether of B-cyclodextrin should correspond to the retention time of the peak of the sodium salt of sulfobutyl ether of B-cyclodextrin in the chromatogram of the comparison solution 3. Characteristic response to sodium |
| 4. | Clarity of solution | Solution should be clear |
| 5. | pH | 4,0-6,8 |
| 6. | Assay | Not less than 95,0% and not more than 105,0% (calculated on dry basis) |
| 7. | Heavy metals | Not more than 5 ppm |
| 8. | Water | Not more than 10% |
| 9. | Related substances: - β – Cyclodextrin - 1,4 – butane sultone - Sodium Chloride -4-Hydroxybutane-1-Sulfonic Acid -Disodium bis- (4-sulfobytil) ether | Not more than 0,1% Not more than 0,5 ppm Not more than 0,2% Not more than 0,09% Not more than 0,05% |
| 10. | Average Degree of Substitution | 6.2-6.9 |
| 11. | Bacterial endotoxins | Less than 0,015 EU/mg |
| 12. | Microbiological purity - Total count of aerobes and fungus in 1 gr - Enterobacteriaceae in 1 gr - Pseudomonas aeruginosa in 1 gr - Staphylococcus aureus in 1 gr | Not more than 102 CFU Absence Absence Absence |
| 13. | Storage conditions | In the protected from moisture place, under temperature not higher than 25 oC |
| 14. | Shelf life | 2 years |